⚙️  d3kOS

Website Integration Requirements

What Must Be Added to d3kOS to Power atmyboat.com

Donald Moskaluk Consulting  ·  March 2026

CONFIDENTIAL — INTERNAL DEVELOPMENT DOCUMENT

# 1.  Overview & Tier-Aware Architecture

Correct Tier Model for Integration T0: Standalone, offline, private. NEVER connects to atmyboat.com. No APIs called, no data transmitted. T1: Free. Connects to atmyboat.com and mobile app. Telemetry push, voyage sync, alerts enabled. Requires account registration. T2: Paid ($9.99/mo). All T1 features plus advanced dashboard features unlocked on web and device. T3: Commercial ($99.99/yr). All T2 plus fleet management, multiple boats, SMS alerts.

## 1.1  T0 — No Integration Required

T0 Privacy Guarantee A d3kOS device in T0 mode must be architecturally incapable of phoning home. No cloud credentials file, no outbound HTTP flows, no registration. This is enforced at the Node-RED flow level — cloud flows are not installed in T0 base image.

- T0 base image ships WITHOUT: cloud-credentials.json, telemetry-push Node-RED flow, alarm-webhook flow, OTA update check service

- T0 image ships WITH: all local services — Signal K, Node-RED dashboard, voice, camera, OpenCPN, onboarding wizard

- T0 → T1 upgrade: user downloads a T1 enablement package from atmyboat.com after registering. This adds the cloud flows to the running system.

## 1.2  Integration Architecture by Tier

| Integration Feature | T0 | T1 (Free) | T2 (Paid) | T3 (Commercial) |
| --- | --- | --- | --- | --- |
| QR Code deep link to register | Generate only (no auto-connect) | Scan → register free account | Same | Same |
| Telemetry push to Supabase | Never | GPS + engine every 60s | GPS + engine every 60s | All boats in fleet |
| Voyage log cloud sync | Never | ✓ on engine stop | ✓ — unlimited history | ✓ — fleet aggregate |
| Helm AI session sync | Never | 30 days | Unlimited | Unlimited |
| Marine Vision cloud upload | Never | Photos | Photos + Video | Photos + Video |
| Outbound alarm webhook | Never | ✓ — push + email | ✓ + SMS | ✓ + SMS + fleet |
| Geofence monitoring service | Never | ✓ | ✓ | ✓ per boat |
| OTA update checker | Manual only | Notification | One-click install | One-click + beta |
| Alert config sync (web → device) | Never | ✓ | ✓ | ✓ |
| Remote diagnostics (T3 future) | Never | Never | Never | Opt-in |

## 1.3  T0 → T1 Upgrade Path on Device

- User creates free account at atmyboat.com

- Scans QR code on n3-k1 (now deep-links to: https://atmyboat.com/register?device=[ID])

- Website issues device API key and boat UUID to the device

- Website sends T1 enablement package URL to device (via d3kOS-update API)

- d3kOS downloads and installs cloud Node-RED flows — no full re-flash needed

- Telemetry push begins within 60 seconds of installation

# 2.  Device Identity & Registration

## 2.1  QR Code Update — Critical

Current QR code encodes a simplified string. Must be updated to a full deep-link URL:

https://atmyboat.com/register?device=[INSTALLATION_ID]&tier=t0&version=[FIRMWARE]

This URL auto-populates the device serial on the website registration form. When scanned by a phone camera, the person lands directly on the T1 registration page with their device pre-identified.

## 2.2  Registration API Endpoints (New on d3kOS)

| Endpoint | Method | Called By | Purpose |
| --- | --- | --- | --- |
| http://d3kos.local:8091/api/link | POST | atmyboat.com post-registration | Receive boat UUID + API key, write to cloud-credentials.json, install T1 flows |
| http://d3kos.local:8091/api/status | GET | atmyboat.com (health check) | Returns: tier, firmware version, uptime, last telemetry push timestamp |
| http://d3kos.local:8091/api/tier/update | POST | atmyboat.com (subscription change) | Update tier on device when user upgrades T1→T2→T3 |
| http://d3kos.local:8091/api/alerts/config | POST | atmyboat.com (alert settings) | Push geofence + alert config from web dashboard to device |

## 2.3  cloud-credentials.json (New File — T1+ Only)

{

"boat_uuid": "[UUID from registration]",

"device_api_key": "[issued by atmyboat.com]",

"supabase_url": "https://[project].supabase.co",

"supabase_anon_key": "[public anon key]",

"webhook_url": "https://atmyboat.com/api/notify",

"tier": "t1",

"registered_at": "2026-03-04T12:00:00Z"

}

Security This file must be in .gitignore, excluded from the base image, and never pushed to GitHub. Created only after successful device registration.

# 3.  Telemetry Push System (T1+ Only)

T0 Enforcement The telemetry push Node-RED flow does not exist in T0 base image. It is installed as part of T1 enablement package only.

## 3.1  Data Flow

NMEA2000 → PiCAN-M HAT → Signal K (port 3000) → Node-RED → [d3kOS-cloud-telemetry-push flow] → Supabase REST API

## 3.2  Telemetry Event Payload

POST https://[supabase-url]/rest/v1/telemetry_events

Authorization: Bearer [DEVICE_API_KEY]

{

"boat_id": "[UUID]",

"event_type": "gps_ping",

"recorded_at": "2026-03-04T14:23:11Z",

"lat": 49.2827, "lng": -123.1207,

"speed_knots": 6.4, "heading_deg": 247.3,

"engine_rpm": 2200, "engine_temp_f": 175.0,

"fuel_level_pct": 68.5, "battery_volts": 12.8,

"fresh_water_pct": 45.0, "black_water_pct": 22.0,

"pi_cpu_temp_c": 52.3, "pi_cpu_pct": 34.0,

"nmea2000_device_count": 4, "internet_connected": true

}

## 3.3  Push Frequency

| Event | Interval | Notes |
| --- | --- | --- |
| Full GPS + engine snapshot | Every 60 seconds | Primary heartbeat when underway |
| Engine metric change | On >5% threshold change | Avoids flooding on stable conditions |
| Alarm event | Immediate | Bypasses queue — priority push |
| Pi health snapshot | Every 5 minutes | System health tracking |
| Voyage start/end | On engine start/stop | Triggers voyage record on server |

## 3.4  Offline Buffer

- SQLite buffer: /opt/d3kos/data/telemetry-buffer.db

- Max 10,000 events (~14 days of GPS pings at anchor)

- On reconnect: flush oldest-first in batches of 100

- T0 devices: no buffer file exists

# 4.  Voyage Log Sync (T1+ Only)

## 4.1  Trigger: Engine Stop

When RPM drops to 0 for 2 consecutive minutes, voyage is finalised and pushed to Supabase:

POST https://[supabase-url]/rest/v1/voyage_logs

{

"boat_id": "[UUID]",

"started_at": "2026-03-04T09:15:00Z",

"ended_at": "2026-03-04T14:40:00Z",

"distance_nm": 42.7, "avg_speed_kts": 7.9,

"fuel_used_gal": 18.4, "engine_hours_added": 5.42,

"route_polyline": "[Google encoded polyline]"

}

## 4.2  Retention by Tier

| Data | T0 | T1 | T2 | T3 |
| --- | --- | --- | --- | --- |
| Voyage logs | Local 30 days only | Cloud unlimited | Cloud unlimited | Cloud + fleet aggregate |
| Boat log voice notes | Local only | Cloud sync | Cloud sync | Cloud sync |
| Engine benchmarks | Local only | Cloud backup | Cloud backup | Cloud backup |

# 5.  Alert & Notification System (T1+ Only)

## 5.1  Alarm Webhook Flow (New Node-RED Flow)

- Listens on Node-RED alarm event topic (already exists in d3kOS)

- On WARNING or CRITICAL: POST to atmyboat.com/api/notify

- Web server sends push notification (all T1+), email (T1+), SMS (T2+)

POST https://atmyboat.com/api/notify

Authorization: Bearer [DEVICE_API_KEY]

{

"boat_id": "[UUID]",

"alert_type": "engine_critical",

"severity": "critical",

"message": "Engine temp 225F, 50 degrees above baseline",

"lat": 49.2827, "lng": -123.1207,

"triggered_at": "2026-03-04T14:23:11Z"

}

## 5.2  Geofence Service (New Python Service — T1+ Only)

- New systemd service: d3kos-geofence.service

- Polls GPS from Signal K every 60 seconds

- Reads geofence config from /opt/d3kos/config/alerts.json

- Calculates Haversine distance from current position to home position

- If distance > radius: POST geofence alarm to webhook immediately

- Suppresses repeat alerts for 30 minutes after first trigger

- NOT installed in T0 base image — T1 enablement package only

## 5.3  Alert Delivery by Tier

| Delivery Method | T1 | T2 | T3 |
| --- | --- | --- | --- |
| Web Push Notification | ✓ | ✓ | ✓ |
| Email | ✓ | ✓ | ✓ |
| SMS (Twilio) | ✗ | ✓ | ✓ — all fleet boats |
| Fleet-wide alert | ✗ | ✗ | ✓ |

# 6.  Marine Vision & Helm AI Sync (T1+ Only)

## 6.1  Marine Vision Upload

- New Python service: d3kos-vision-upload.service (T1 enablement package)

- Scans /opt/d3kos/data/camera/ every 30 minutes when connected

- Uploads photos to Supabase Storage (T1+); video additionally for T2+

- Posts metadata including YOLOv8 detection tags to marine_vision_media table

- Local 7-day cleanup continues regardless of upload status

## 6.2  Helm AI Session Sync

- Modify services/voice/assistant.py: log each session turn to /data/helm-sessions/

- New Node-RED flow: d3kOS-helm-session-upload

- Watches for completed session files, pushes to helm_ai_sessions table

- Retention: T1 — 30 days cloud; T2/T3 — unlimited

# 7.  OTA Update System

## 7.1  Update Manifest (Hosted on atmyboat.com)

GET https://atmyboat.com/api/d3kos/releases/latest.json

{

"version": "2.1.0", "release_date": "2026-05-01",

"download_url": "https://cdn.atmyboat.com/d3kos-v2.1.0-update.tar.gz",

"sha256": "[checksum]", "is_critical": false

}

## 7.2  Update Checker Service

- New systemd service: d3kos-update-checker.service (all tiers — T0 included)

- Runs daily at 03:00 when internet available (T0 checks manually via Settings UI)

- If newer version: show update badge on dashboard header

- T0: manual download link shown; T1: auto-notification; T2/T3: one-click install

## 7.3  One-Click Install (T1+ Only)

- 'Install Update' button in Settings → download → verify SHA256 → apply update script

- Updates service files and scripts only — no full SD card re-flash needed

- Post-update: pushes version confirmation to web dashboard

# 8.  New Files for d3kOS Repository

## 8.1  New Files

| File Path | Type | Tier | Purpose |
| --- | --- | --- | --- |
| services/cloud/telemetry-push.json | Node-RED flow | T1+ | Telemetry push to Supabase |
| services/cloud/alarm-webhook.json | Node-RED flow | T1+ | Outbound alarm notifications |
| services/cloud/voyage-sync.json | Node-RED flow | T1+ | Voyage log push on engine stop |
| services/cloud/helm-session-upload.json | Node-RED flow | T1+ | Helm AI session sync |
| services/geofence/geofence.py | Python service | T1+ | Geofence monitoring |
| services/vision/vision-upload.py | Python service | T1+ | Marine Vision cloud upload |
| services/ota/update-checker.py | Python service | All | OTA version check |
| services/ota/apply-update.sh | Bash script | T1+ | Apply downloaded update |
| config/cloud-credentials.json | Config (gitignored) | T1+ | Device API key, Supabase URL, boat UUID — created post-registration |
| config/alerts.json | Config | T1+ | Geofence + alert settings synced from web |
| etc/systemd/system/d3kos-geofence.service | systemd unit | T1+ | Geofence service |
| etc/systemd/system/d3kos-vision-upload.service | systemd unit | T1+ | Vision upload service |
| etc/systemd/system/d3kos-update-checker.service | systemd unit | All | OTA check service |

## 8.2  Files to Modify

| Existing File | Change Required |
| --- | --- |
| services/onboarding/ (QR generation) | Update QR URL to: https://atmyboat.com/register?device=[ID]&tier=t0&version=[VER] |
| services/voice/assistant.py | Add session start/end logging to /data/helm-sessions/ |
| opt/d3kos/config/license.json | Add fields: cloud_registered (bool), boat_uuid, tier_on_device |
| opt/d3kos/config/engine-manufacturers.json | Add version field; expose via atmyboat.com/api/engines public endpoint |
| README.md | Add T0/T1/T2/T3 tier explanation; add T1 registration instructions; correct hardware name to n3-k1 |
| MASTER_SYSTEM_SPEC.md | Add Section 13: Cloud Integration Architecture for T1+ tiers |

## 8.3  Implementation Priority

| Feature | Effort | Priority | Phase |
| --- | --- | --- | --- |
| QR code deep link update | 1 day | Critical | 1 |
| Registration handshake API | 2–3 days | Critical | 1 |
| cloud-credentials.json + auth | 1 day | Critical | 1 |
| Telemetry push Node-RED flow | 3–4 days | Critical | 1 |
| SQLite offline buffer | 2 days | High | 1 |
| Voyage log sync on engine stop | 2 days | High | 1 |
| Alarm webhook outbound | 2–3 days | High | 2 |
| Geofence service (new) | 3 days | High | 2 |
| Alert config sync (web → device) | 1–2 days | High | 2 |
| OTA update checker | 2–3 days | High | 2 |
| One-click update (T1+) | 4–5 days | High | 2 |
| Helm AI session logger + upload | 2 days | Medium | 2 |
| Marine Vision upload service | 3 days | Medium | 3 |
| T1 enablement package system | 2–3 days | High | 1 |

Phase 1 Estimated Total 12–15 developer days to achieve a working T1 member dashboard: device registers, telemetry flows, voyage logs appear, GPS map shows last position, alerts fire.

# 9.  Security

## 9.1  Key Principles

- T0: no cloud code exists on device — cannot accidentally connect

- All outbound calls: HTTPS only, certificate verification enforced (Python requests verify=True)

- Device API key: write-only to telemetry tables — cannot read other boats' data

- Supabase Row Level Security (RLS): each device writes only to its own boat_id

- cloud-credentials.json: in .gitignore, never in base image, GPG encrypted at rest

- Local API ports (8084–8101): bind to LAN only, not exposed to internet

## 9.2  Device Deactivation (Stolen / Sold)

- Owner: Dashboard → My Boats → Deactivate Device

- Sets boats.is_active = false in Supabase — pushes rejected with 403

- Revokes device API key — all future pushes return 401

- Device continues offline (T0 behaviour) — cloud sync simply stops

- New owner must factory reset and re-register to link to a new account